# Safety concept and terminal map

Draft for supervisor approval. The kit is a PLC, a VFD and the software, used in EPAC lab sessions. The lab supplies the motor, contactors and e-stop. Students connect them to the terminals listed in section 4.

## 1. Principle

A student never touches a live conductor. Terminals wired while the supply is on carry extra-low voltage or are isolated from the mains. Terminals that carry mains or motor voltage are shrouded and wired only with the supply off and locked.

## 2. Hazards and measures

| Hazard | Measure |
| --- | --- |
| Live mains or motor terminals | Shrouded, finger-safe terminal blocks, color-coded and separated from the control terminals by a barrier. Wiring only with the supply off and locked. |
| Charge left on the DC bus after power-off | Bus-live LED on the panel. Bleed resistors bring the bus below 50 V in about 2 minutes. Label: wait 3 minutes before rewiring. |
| VFD logic ground at DC bus potential | Closed enclosure, opened with a tool by the instructor only. Every student terminal is isolated: RS485, encoder, enable, analog. |
| Earth fault | PE terminal. Heatsink and enclosure bonded to PE. RCD on the bench feed, to be confirmed by the lab. |
| Unexpected motor start | External enable input (dry contact) in the VFD enable path. Faults are latched with no automatic restart. A new run command is required after the enable closes. |
| Overcurrent and short circuit | Short-circuit trip in the IPM (hardware), software overcurrent trip, T10A fuse on the mains input. |
| DC bus overvoltage when decelerating | Overvoltage trip at about 375 V. Deceleration ramp limited, since there is no braking resistor. |
| Loss of the Modbus link | The VFD ramps down after 500 ms without a valid frame, then latches a fault. |
| Mains on the PLC relay outputs | Relays rated 250 Vac, 5 A, isolated contacts, at least 6 mm creepage to the logic side, rating printed beside each terminal. Decision pending, see section 6. |
| Access to the power section for debugging | Probing, USB debug and scope work are instructor-only, behind an isolation transformer. |

## 3. Working rules

| # | Working rule |
| --- | --- |
| 1 | Supply off and locked before any wiring on a power terminal. |
| 2 | Wait 3 minutes after switching off and check that the bus-live LED is out. |
| 3 | Wire from the terminal map only. Connect nothing to a terminal that is not listed. |
| 4 | The instructor checks the wiring before the first power-up. |
| 5 | Test the e-stop at the start of each session. |
| 6 | Students do not open the VFD enclosure. |
| 7 | A fault is cleared only after its cause is found. |

## 4.1 VFD power terminals

| Terminal | Function | Rating | Isolated |
| --- | --- | --- | --- |
| L, N | Single-phase mains input, through a T10A fuse | 220 V AC, 50 Hz | No |
| PE | Protective earth for enclosure and heatsink | Connect to bench earth | n/a |
| U, V, W | Motor output, 3-phase | Motor up to 0.75 kW, about 3.5 A rms; hardware trip about 10 A | No |
| PE (motor) | Motor frame earth | Connect to the motor frame | n/a |

## 4.2 VFD control terminals

| Terminal | Function | Rating | Isolated |
| --- | --- | --- | --- |
| EN1, EN2 | Enable input, dry contact. Closed = run allowed | Internal isolated 24 V, about 10 mA | Yes |
| RS485 A, B, GND | Modbus RTU slave, link to the PLC | 115200 baud | Yes |
| ENC +5V, 0V, A+, A-, B+, B-, Z+, Z- | Incremental encoder input, differential or single-ended | 5 V, about 100 mA supply | Yes |
| AIN V, AIN I, AGND | Process feedback, 0-10 V or 4-20 mA (only if the isolator module is kept) | 0-10 V or 4-20 mA | Yes |

## 4.3 PLC terminals

| Terminal | Function | Rating | Isolated |
| --- | --- | --- | --- |
| +24V, 0V | PLC supply | 24 V DC, 1 A | Own domain |
| DI0-DI7, COM | Digital inputs | 24 V DC, about 5 mA each | Yes |
| DO0-DO5 | Relay outputs, one normally open contact each | 250 Vac / 5 A if mains-rated (pending), else 30 V max | Yes |
| COM1 A, B, GND | RS485 port 1, master to the VFD | 115200 baud | No, isolated on the VFD side |
| COM2 A, B, GND | RS485 port 2, slave to the laptop through an isolated USB-RS485 adapter | 115200 baud | No, isolated in the adapter |

## 5. Provided by the lab

| Item | Requirement |
| --- | --- |
| 3-phase motor | 0.37 to 0.75 kW, 220 V winding (delta), guard over the coupling and any load |
| Contactor | Rated for the motor. Coil voltage must match the relay output decision in section 6 |
| E-stop | Normally closed contact, wired to EN1/EN2 and to the contactor coil circuit |
| Bench feed | Earthed, fused, with an RCD |
| Isolation transformer | For instructor bench work. Needed for students only if the supervisor requires it |

## 6. Open points for the supervisor

| Question for the supervisor | Effect on the design |
| --- | --- |
| Does EPAC require an RCD, a contactor or an enclosure rating for lab equipment? | May add parts and change the enclosure choice |
| PLC relay outputs: mains-rated or 24 V only? | Mains-rated needs slots and creepage on the PLC board. 24 V only limits the contactors students can use |
| Is an isolation transformer available, and is it required for student use? | If required, it is an extra item per VFD station |
| How many kits, and how many students per session? | Decides the kit count and whether PLC boards are separate from the VFD station |
| Is the external enable input on the VFD required? | Adds one isolated input and a hardware fault path on the G474, to be added to the schematic |